| POKOLBIN | PENRITH | MACKAY |
| --- | --- | --- |
| REMOTELY SUPERVISED CONTINUOUS MINER |  |  |
| Must Haves - Sensing, sensing, sensing – IRGPS, Ranging, drill monitoring and tramming feedback, INS. - Simultaneous activated automated bolters and roof containment handling. - Remote station operation console – e.g.: crib room which needs to be advanced with panel - Extensible services on haulage system including ventilation. - FCT – automated haulage. - Steering sensing – Horizon control gamma ray detection, vision system. - 3D simulation and CAD/CAM – e.g. Delmia - On-line laser profiling of roof and ribs for real time roadway conditions. - Onboard computers with communication with other systems eg. Haulage systems. - Consumable feeding system either onboard or as part of extensible service system. - Ability to manual control. - Sumping head. - Training for both operators and service tradesman. Metrology - Powerpoint slide Indoor IRGPS - Powerpoint slide | Operating Scenario - Outburst/inrush mining - Increase productivity - Parallel tasks - Reduced manning levels - Reduced ventilation requirements - Minimum distance 250 m between miner and remote operating station What do We Want!! - Improved operator safety and health (dust, noise, heat, humidity) - High/good visibility - Automated roof support system - Access/ information to enable good horizon control - Roadway driven on centre - Data monitoring  environment, machine health, machine function - Simple to use remote operating stations (ROS) - Bullet proof standards and procedures - Match supply chain to fully automated roof support system - Match the capability to the mine design and plan Overview - Miner fitted with CCTV - Operator positioned minimum 250 m away - Observes/controls miner via CCTV - Head, tail, horizon control, machine diagnostics visible - Auto re-supply of consumables for automated bolters - Safe repeatable process | Functional Specification - Knows where it is - Landmark system (shearers) - Recognise changing conditions and adapt (roof support, etc) - Operate at 10 MPOH - Adaptable to good and bad strata conditions - All functions mechanised/automated - Remote location control - Nil ventilation at face - Inert atmosphere - Use in conjunction with continuous haulage and service monorail systems - Autobolting systems - Robust, 2-way communication linkage - Advanced self diagnostics - Low ground pressures - Break-off ability - Auto cut out when people enter mining place or machine gets out of control - Web-cam of cutting operation |

| POKOLBIN | PENRITH | MACKAY |
| --- | --- | --- |
| AUTOMATED INSTALLATION OF ROOF AND RIB SUPPORTS |  |  |
| Functional Specification - Simplified - User Friendly - Modular – bulk lots onto miner - Miner not designed with bolting in mind - Mesh handling - Automated bolting rigs - Grout tank in pods with bolts - Bulk resin supplied in pipes on continuous conveyor/monorail - Purpose built bolting machine that also cuts coal - Pod exchange in conjunction with FCT - Support mapping – GPS – laser - Monitor TRS height and auto adjust bolting pattern - Bad ground detection – auto PUR or Rocksill - Improved consumable handling - Face/advance drilling on continuous miner - Spray on skin protection – especially ribs - Transparent spray on skin - Programmable sequence – breakaways, etc - Automatic stonedusting - Reliability Bolting Process Head cuts and loads onto FCT  Auto polymer skin sprayed behind curtains  Auto bolts installed  Resin injected  Advance  Repeat as necessary Monitor, Control, Measure | Parameters - Should cater for a range of different support types including cables and bolts - Should meet all geological requirements - Fits in range of heights and widths - Easy to supply without manual labour - Easy to program - Fail to safe mode - Ergonomic to maintain - Maps roof profile – adaptive to conditions - Maps strata as holes are drilled - Ability to install tell-tales - Remote operation from a safe area - Full surface monitoring - Diagnostics - No of bolts, position of bolts - Geological data - Status of consumables | Functional Specification - Install 1 sheet of mesh in 6 minutes  10 MPOH - Install 6 bolts in 6 minutes - Continuous supply feed to miner – not batch - Supply outbye - Manual override on all automation - Must have continuous monitoring and report on automation to control centre - Must be adaptable to most conditions  breakaways! - Integrated to the whole system: cut  load  supply - IS or flameproof - Total cycle < 6 minutes What We Want - Robust, tried and true - Standardised equipment - Roof mapping – artificial intelligence (AI) so that it can respond to the situation and change the conditions - Retro-fitting to current generation machines - Simple maintenance - Flow Open Issues - Continuous supply over batch load  lateral - Swap between support types 2.1 m  4.0 m  8.0 m - Extreme conditions - Face to support distance - TRS – longwall shields over the shearer |
| CONTINUOUS HAULAGE |  |  |
| Operating Scenario - To safely, efficiently and cost-effectively transport a minimum of 10m cut coal for 20 hours in each 24 hour period What Does it Need to Do - Carry panel services - Air - Water - Pump Out - Grout - Ventilation - Oil - Stonedust - Cables - Hydraulic Bolting Rigs on conveyor - Ability to build structure without stopping panel belt - Self-tracking - Easily Retracted - Monorail services from Boot to TX - Outbye services - Air - Water - Power - Communications - Consumables Open Issues - Light-weight services and structure - Miner easily able to carry enough supplies for sequence - 3.3kV Miner/Bolter - Crew remote operation - Phone Boxes Required for Superman Suit Changes Flowchart of Process - Cut a pillar in 20hrs - Maintain for 4hr period per 24hrs - Every 3rd day power/services move - Plug-and-Play Phones, Communications etc - Pre-made services (phones, services etc) | Functional Specification - High availability (99%) - Easy operation - User friendly - Design integrated for further operational use eg; longwall monorail - Must be able to extend to 210 m - Must be capable of turning 900 left or right twice in length of system - Minimal manning (1 person) - Working height – 1.8 m to 4.0 m - Able to meet peak loading capacity of current continuous miners - Services integrated into continuous haulage system - Maintainability - Ease of maintenance - Fit for purpose - Easy access - Durability and extended service life - Easy relocation process - Minimal time to relocate - Hopper to take 3 shuttle cars - Air cooled – minimal water usage - Capability to handle changing mining conditions - Sloppy floor - Uneven floor - Poor roof - Grades up to 200 - Compatible to current underground power supply (1000v) - Approved system in line with underground regulation in NSW - Design risk assessment - Operational risk assessment - Training package – mechanical, electrical, operators - Continuous OEM support for ramp up process - Back up support by OEM in service, support and critical parts - Minimal spillage - Easy flit, minimal duration for flit - Must comply with transport envelope | Operating Scenario - To safely, efficiently and cost-effectively transport a minimum of 10m cut coal for 20 hours in each 24 hour period What Does it Need to Do - Carry panel services - Air - Water - Pump Out - Grout - Ventilation - Oil - Stonedust - Cables - Hydraulic Bolting Rigs on conveyor - Ability to build structure without stopping panel belt - Self-tracking - Easily Retracted - Monorail services from Boot to TX - Outbye services - Air - Water - Power - Communications - Consumables Open Issues - Light-weight services and structure - Miner easily able to carry enough supplies for sequence - 3.3kV Miner/Bolter - Crew remote operation - Phone Boxes Required for Superman Suit Changes Flowchart of Process - Cut a pillar in 20hrs - Maintain for 4hr period per 24hrs - Every 3rd day power/services move - Plug-and-Play Phones, Communications etc - Pre-made services (phones, services etc) |
| PANEL ADVANCE |  |  |
|  | Functional Specification - Total service system - Ventilation - Air - Water - Power - Communication (Citech) - Miner service system - Supplies - Spray on liners - Dust - Grout - Resin - Minimal operator interface - Prefer fully autonomous - Simple, simple, simple - Replacement for 6” Victaulic pipes - Light weight materials - FRAS Poly  pressure rating - 100 m rolls  H/T reelers - Bulk consumables  liquid/dust - Integration with continuous miner Other Issues - Live belt moves  TBMs! - Stoppings - Low labour - No dust - Fast - Rating fit for purpose - Improved pumping - Coal slurry  flocc out - Dewatering fines - Roadway consolidation – replace skulls - Bulk supplies - Max QA pod - Return to rail mounted systems (Gujarat) - Self building railway technology | What is the size of the problem? - CM2010 = 1,000 metres per week = 400 linear metres per week - That means - 1,200 m of pipes (water, air, waste) - 400 m of H/T cable - 8,000 bolts - 1,000 sheets of mesh - 400 m of conveyor structure - 800 m of conveyor belting - 400 m of fibre optic cable - Fire and escape equipment - Heaps of stonedust - 1,000 m of exposed road surface to maintain Open Questions - Why stay with 100 m pillars? - What else can we use for pipes? - How do we make monorail stretch? - Limited by voltage drop? - How do we get enough people to do all the work? - How do we manage the gas make and rib emissions? |
| ENGINEERING RELIABILITY |  |  |
| Functional Specification - Consistent Reporting - Standards and Definition - Machine flagging that it needs work - Pre-warning that work required - Prioritising work required - Engineer over-ride for planning purposes - Machine diagnostics within set parameters - Live monitoring to the surface - Operator and trades skill levels to minimum standard or higher - Remote diagnosing and support repair | Fit for Task - Correctly specified Improvement - More maintenance friendly - Too technical – simple is better - Train or better training for trades and operators - Redundant systems - Quick replacement/changeout - Plan for continual improvement - Particularly on overhauls - Common parts Operating Scenario - Cut coal, bolt, convey - 20 hours per day - Continual services (water, hydraulic, electrical) - Outbye hydraulics and resin tanks Diagnostics |  |

| POKOLBIN | PENRITH | MACKAY |
| --- | --- | --- |
| PLANNING ORGANISATION, PROCESS CONTROL |  |  |
| Goals - Longer term planning horizon - Geology - Mine design - Better predictive tools - Effectiveness of long term plan - Fit for purpose equipment and manning Planning - Standards for each development metre - Short interval process control - After action reviews - Incorporating risk - Predictive tools - Simplicity - Visual planning indicators - Communication strategy - Defined place for everything, and everything in its place Organisation - Resource levelling - Understanding responsibilities - Communication Process Control - Better visual systems - Reduce variability - Automatic updates - Actual versus planned - Continually challenge work methods - Auditing process - Feedback loops - Engineer out difficult and dangerous processes - Improve proficiency - Training – virtual reality |  |  |
| PEOPLE BEHAVIOUR AND SKILLS |  |  |
| Motivation/Job Satisfaction - Involvement with all crew members - Incentives/rewards: - Money - Field trips - Encouragement – positive feedback - Lifestyle: - Rosters - Location - Shift lengths - Career path - Create opportunities for change of programs - Exposure to other operations - Newer/repaired equipment - Metrics and actual performance clearly visible to all Suitable Role - Selecting correct person for the job - Past experiences - Correct behaviour pattern - Relative skills - Positive attitude - People management skills Skills - Regular training - Identifying skills shortages - External skills training - Performance reviews Roles/Responsibilities - Management plans - Knowing the role being undertaken - Setting expectations - Performance reviews Providing Reliable Equipment - Take more care of equipment - Operator training - Management plan to retain trades based skills - Standard operating procedures - Schedule routine changeout/timeframe Standards Between Shifts - Communications – talk about shifts problems - Standardised cutting procedure, using training and auditing - Panel standard management plan/sign off when complete - Crew changeover talk - Auditing - Continued training Communication - Crew talks - Tool box talks - Pre/post shift briefings - Safety alerts - Management presence underground PEOPLE WITH CREDIBLE, CONSISTENT EADERSHIP | Current Scenario - Workforce that is highly dependant upon structure and compliance - Entrenched culture - Skills shortage Where Do We Want To Be? - Coal miners that have training, skills and experience to operate in modern technologically advanced development panels Barriers At Present - High turnover, skills shortage - Lack of will and expertise in middle and front line management - Training doesn’t meet expectations - Culture in place at some sites that doesn’t promote continuous improvement How do We Overcome This? - Effective leadership training - Operator tailored training  behavioural - Retention strategies - Lifestyle - Roster - Development strategies - Selective recruitment based upon team composition |  |
| WORKSHOP CRITIQUE |  |  |
|  |  | - Incorporate major workshop component rather than as add-on at end of day - Nominate syndicate leaders and tasks earlier in day - More preparation time - How to achieve parallel tasks - What is the vision and objective of the Roadway Development Operators’ Workshops? |